# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of equipment for transport of waste from illegal dumps to legal landfills 	p 1 /…

Publication reference : RD-02-29-252/22.11.2016/PP2/T4

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

All functional and technical specifications are to be considered minimum and mandatory for all equipment and are cumulative, unless otherwise specified. A single product must comply with all specifications listed in its category/section simultaneously, to be considered compliant. All products must have at least 2 (two) years warranty if not specified otherwise. Refurbished, used or pre-owned components, devices or equipment are not accepted. For all components, products with either equivalent or exceeding specifications are accepted, but bidders must provide valid proof for each claim. The bidder must ensure that by including products and services equivalent or exceeding the specifications do not break the functionality and / or the integrity of the final solution.

Memorandum of the Tenderer must contain description of the technical characteristics related to offered item, which must at least contain technical characteristics requested by the Contracting Authority, signed by the authorized person and certified by stamp.

Manufacturer of the chassis truck and truck upgrade must contain certification about required technical characteristics.

Authorization for tender participation given by manufacturer of the chassis truck and truck upgrade, which will prove originality of goods, and right of the tenderer to offer and sell offered items.

List of authorized services for offered items in Republic of Serbia, which is certified by the manufacturer of the offered goods, with addresses and contact phones ( for chassis and truck upgrade)

Description of the technical characteristics, the manufacturer's certificate, the authorization and the list of authorized services must be original and signed by the responsible person of the manufacturer and certified with the stamp of the manufacturer. If the said documentation is provided in a foreign language, it must be translated into Serbian and verified by a court interpreter. The offered chassis and truck upgrade must fulfil all the requirements stipulated in the technical specification from the tender documentation, otherwise the offer will be rejected as unacceptable.

If the tenderer is simultaneously the manufacturer of the chassis or truck upgrade that is offered, the Authorization referred to in point 3. delivers in the form of Statements under material and criminal responsibility that he is the manufacturer of the offered chassis / upgrade and signed by an authorized person and stamped.

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
|  | Equipment for transport of waste from illegal dumps to legal landfills 4pcs |  |  |  |
|  | Gross vehicle weight min. 26.000 kg |  |  |  |
|  | Engine power min. 220kW- Diesel |  |  |  |
|  | Engine torque min. 1.300 Nm |  |  |  |
|  | Servo steering |  |  |  |
|  | Cabin capacity min. 2 persons |  |  |  |
|  | Number of doors: 2 |  |  |  |
|  | Safety: |  |  |  |
|  | ABS |  |  |  |
|  | AEBS Advanced emergency breaking system |  |  |  |
|  | Alarm for reverse moving |  |  |  |
|  | Digital tachograph for two drivers |  |  |  |
|  | Accompanying equipment: spare tire, wheel wrench, crane for mounting a wheel, triangle |  |  |  |
|  | Air condition in cabin |  |  |  |
|  | Front under run protect |  |  |  |
|  | Fog lights |  |  |  |
|  | Electronic windows |  |  |  |
|  | Wheelbase min 3.450 mm |  |  |  |
|  | Length min 5.943 mm |  |  |  |
|  | Maximum height min (unloaded) 2.276 mm |  |  |  |
|  | Front track min 1.724 mm |  |  |  |
|  | Back track min 1.542 mm |  |  |  |
|  | Front leash min 1008 mm |  |  |  |
|  | Back leash min 1.485 mm |  |  |  |
|  | Height at the end of the chassis max 771 mm |  |  |  |
|  | Width of the vehicle min 2.010 mm |  |  |  |
|  | Maximum permissible mass 5.200 kg |  |  |  |
|  | Front axle load min 2.100 kg |  |  |  |
|  | Back axle load min 3.700 kg |  |  |  |
|  | Mass of the chassis 2.049 kg ± 2% |  |  |  |
|  | Useful payload of the chassis 3.151kg ± 2% |  |  |  |
|  | Technical characteristics of the upgrade for garbage trucks for the manual garbage disposal: |  |  |  |
|  | Volume of the waste bin (collector without the receiver) must be at least V = 5m 3 . |  |  |  |
|  | Body of the truck upgrade must be waterproof. |  |  |  |
|  | Volume of the garbage dump - minimum V = 1.0 m 3 . |  |  |  |
|  | Controls (management of truck upgrade functions) – hydraulic-electric. |  |  |  |
|  | Compaction of garbage - the degree of compression must be in at least size 1: 3. |  |  |  |
|  | Command for opening backdoor is located on the front load compartment. |  |  |  |
|  | Commands for truck upgrade are located in the back of the truck, on its sides. |  |  |  |
|  | Commands type - single stroke and one automatic cycle. |  |  |  |
|  | Maximum operating pressure in the system for the exploitation of the vehicle must be maximum of 120 bar. |  |  |  |
|  | Hydraulic system must be secured against overload via safety valves. |  |  |  |
|  | Built-in piston axial pump (flow rate approx 50cm3 / o), the pump is installed directly on the vehicle&#39;s power take-off, without cardan |  |  |  |
|  | Two platforms – place to stand and handrails on the rear end of the truck for safe transport for workers. |  |  |  |
|  | Filter with return water thermometer. |  |  |  |
|  | Blocked valve on the door cylinders to prevent accidents. |  |  |  |
|  | Place of the garbage loading is in the back of the garbage truck. |  |  |  |
|  | Lighting of the work space is in the back of truck – back headlight. |  |  |  |
|  | Rotating lights are located on front and back sides of the truck. |  |  |  |
|  | Anti-corrosive protection and finishing paint: All the elements are painted, painted with base paint and finishing layer with two component acrylic paint, colour dried in chamber with high temperature. RAL color optional, at customer’s request. |  |  |  |
|  | System with receiving and discharging type named ”push plate” with rotating bin for garbage collection and rotary plate for pressing the garbage. |  |  |  |
|  | Controlled discharge drain for liquid flow is located in truck upgrade and collector basket. |  |  |  |
|  | Lifting / lowering the truck doors and locking / unlocking truck doors on samcommand |  |  |  |
|  | Lock the truck door automatically when closing the door. |  |  |  |
|  | Camera placed in the back of the truck and monitor in the cabin, panic STOP button on both sides of the truck upgrade (back of the truck) and in the cabin of the vehicle. |  |  |  |
|  | Certificates |  |  |  |
|  | Production plant certificated against EN ISO 3834-2 or equivalent |  |  |  |
|  | Certified welders with the certificate EN ISO 9606-1 or equivalent |  |  |  |
|  | WPMI label (International Identification Number for the Suppliers) for the truck upgrade manufacturer or equivalent |  |  |  |
|  | SRPS ISO 9001: 2008 or equivalent for the production of truck upgrades |  |  |  |
|  | SRPS ISO 14001: 2015 or equivalent for the production of truck upgrades |  |  |  |
|  | SRPS OHSAS 18001: 2008 or equivalent for the production of truck upgrades |  |  |  |
|  | All requirements mentioned and requested in this document are accompanied by the mention “or equivalent”. |  |  |  |
|  | All requirements mentioned and requested in this document are minimal and mandatory. |  |  |  |